# Prompt: Build a Showcase Prototype for an AI-Assisted PCB Design Generator

## Project title

AI-Assisted PCB Design Generator with Controlled Sensor Selection

## Project goal

Build a showcase prototype of an AI-powered PCB design generator.

The prototype should demonstrate that a user can describe a small electronic board in natural language, and the system can convert that requirement into a structured PCB design output.

This is not a full production PCB design system. It is a controlled feasibility prototype.

The system should use one fixed PCB architecture and allow a small number of controlled sensor substitutions. The goal is to make the prototype feel dynamic while keeping the design simple, reliable, and easy to build.

# 1. Main idea

The prototype should work like this:

User Input

→ AI extracts requested sensor requirements

→ System maps requirements to supported components

→ System generates fixed architecture with selected sensors

→ System displays PCB design output

The PCB architecture remains fixed.

Only the sensor selection changes.

# 2. Fixed PCB architecture

The board architecture must always be:

USB-C 5V Input

→ 3.3V Voltage Regulator

→ ESP32-WROOM-32 MCU

→ Shared I2C Sensor Bus

→ Selected Sensors

→ Status LED / Reset Button

The ESP32-WROOM-32 is suitable because it is a WiFi, Bluetooth, and Bluetooth LE MCU module. (Arrow)

For USB-C power-only operation, the prototype should show CC1 and CC2 pull-down resistors using 5.1kΩ resistors so the board behaves as a USB-C sink device. (Altium)

The sensors should share the same I2C bus. For the demo, use ESP32 GPIO21 as SDA and GPIO22 as SCL. (Cirkit Designer)

# 3. Supported user requirements

The prototype should support only these controlled sensing categories:

| User requirement | Component selected by system |
| --- | --- |
| Temperature | AHT20 |
| Humidity | AHT20 |
| Light | BH1750 |
| Air quality / VOC | SGP30 |
| Pressure | BMP280 |

Temperature and humidity should map together to AHT20.

The user does not need to know or provide the component names. The user only gives requirements.

# 4. Exact user inputs supported by the prototype

The prototype should support these example inputs:

## Example Input 1

Make me a USB-C powered indoor monitoring board with WiFi, Bluetooth, temperature, and humidity sensing.

Expected selected sensors: JSON

{

"selected_sensors": ["AHT20"]

}

## Example Input 2

Make me a USB-C powered indoor monitoring board with WiFi, Bluetooth, temperature, humidity, and light sensing.

Expected selected sensors: JSON

{

"selected_sensors": ["AHT20", "BH1750"]

}

## Example Input 3

Make me a USB-C powered environmental monitoring board with WiFi, Bluetooth, temperature, humidity, light, air quality, and pressure sensing.

Expected selected sensors: JSON

{

"selected_sensors": ["AHT20", "BH1750", "SGP30", "BMP280"]

}

# 5. Exact output the prototype must generate

After the user enters a supported prompt, the system must generate the following output sections.

## Section A: Original User Requirement

Show the user’s input exactly as entered.

Example:

Make me a USB-C powered indoor monitoring board with WiFi, Bluetooth, temperature, humidity, and light sensing.

## Section B: Parsed Requirement

Show the AI-extracted meaning.

Example: JSON

{

"power_input": "USB-C 5V",

"wireless": ["WiFi", "Bluetooth"],

"mcu": "ESP32-WROOM-32",

"logic_voltage": "3.3V",

"communication_bus": "I2C",

"requested_sensing": ["temperature", "humidity", "light"],

"selected_components": ["AHT20", "BH1750"]

}

## Section C: Generated System Architecture

Show this block diagram:

USB-C 5V Input

↓

3.3V Regulator

↓

ESP32-WROOM-32

↓

Shared I2C Bus

↓

Selected Sensors

For the selected sensor example:

USB-C 5V Input

↓

3.3V Regulator

↓

ESP32-WROOM-32

↓

I2C Bus

├── AHT20: Temperature + Humidity

└── BH1750: Light

## Section D: Generated Component List

For every output, always include the fixed base components:

| Component | Purpose |
| --- | --- |
| USB-C receptacle | 5V power input |
| 2 × 5.1kΩ resistors | USB-C CC1/CC2 sink configuration |
| 3.3V voltage regulator | Converts USB-C 5V to 3.3V |
| ESP32-WROOM-32 | Main controller with WiFi and Bluetooth |
| 2 × 4.7kΩ resistors | I2C SDA/SCL pull-ups |
| Decoupling capacitors | Power stability |
| Status LED + resistor | Visual board status |
| Reset button | Basic user control |

Then include only the selected sensor components.

Example for temperature, humidity, and light:

| Selected sensor | Purpose |
| --- | --- |
| AHT20 | Temperature and humidity sensing |
| BH1750 | Ambient light sensing |

## Section E: Generated Netlist-Style Connection Summary

Always show the fixed power and MCU connections:

USB_C.VBUS → REGULATOR.VIN

USB_C.GND → SYSTEM.GND

USB_C.CC1 → 5.1kΩ → GND

USB_C.CC2 → 5.1kΩ → GND

REGULATOR.VOUT → 3V3_RAIL

REGULATOR.GND → SYSTEM.GND

3V3_RAIL → ESP32.3V3

SYSTEM.GND → ESP32.GND

ESP32.GPIO21 → I2C_SDA

ESP32.GPIO22 → I2C_SCL

I2C_SDA → 4.7kΩ → 3V3_RAIL

I2C_SCL → 4.7kΩ → 3V3_RAIL

Then add selected sensor connections.

For AHT20:

3V3_RAIL → AHT20.VCC

SYSTEM.GND → AHT20.GND

I2C_SDA → AHT20.SDA

I2C_SCL → AHT20.SCL

For BH1750:

3V3_RAIL → BH1750.VCC

SYSTEM.GND → BH1750.GND

I2C_SDA → BH1750.SDA

I2C_SCL → BH1750.SCL

For SGP30:

3V3_RAIL → SGP30.VCC

SYSTEM.GND → SGP30.GND

I2C_SDA → SGP30.SDA

I2C_SCL → SGP30.SCL

For BMP280:

3V3_RAIL → BMP280.VCC

SYSTEM.GND → BMP280.GND

I2C_SDA → BMP280.SDA

I2C_SCL → BMP280.SCL

## Section F: Generated Schematic Output

The prototype should display a schematic-style diagram.

For the showcase, this can be one of the following:

- A static schematic image for each supported sensor combination.

- A dynamic diagram rendered from JSON.

- A simplified SVG or Mermaid diagram.

The schematic must show:

USB-C

→ Regulator

→ ESP32

→ I2C selected sensors

Sensors not requested by the user should either:

- not appear, or

- appear greyed out as “not populated.”

The preferred demo style is:

Selected sensors = highlighted

Unselected sensors = greyed out / not populated

## Section G: Generated PCB Layout Output

The prototype should display a PCB layout-style image.

For the first prototype, the PCB does not need to be truly regenerated every time. Instead, use a fixed board layout with all possible sensor footprints.

The system should highlight only the selected sensors.

Example:

Base board:

USB-C + regulator + ESP32 + I2C bus + four sensor footprints

If user asks for temperature and light:

AHT20 and BH1750 are highlighted

SGP30 and BMP280 are greyed out as not populated

This makes the demo look dynamic while keeping implementation simple.

## Section H: Design Explanation

Generate a short explanation.

Example:

The generated board uses USB-C as a 5V input and converts it to a 3.3V logic rail for the ESP32 and sensors. The ESP32-WROOM-32 provides WiFi and Bluetooth connectivity. The selected sensors communicate over a shared I2C bus, which allows the board to support different sensor combinations without changing the main architecture. For this request, the system selected AHT20 for temperature and humidity sensing and BH1750 for light sensing.

# 6. AI behavior

The AI/LLM should only perform controlled requirement extraction.

It should not invent new sensors, new chips, or new architectures.

The AI must output a structured JSON object like this: JSON

{

"power": "USB-C 5V",

"connectivity": ["WiFi", "Bluetooth"],

"requested_sensors": ["temperature", "humidity", "light"],

"selected_components": ["AHT20", "BH1750"],

"unsupported_requirements": []

}

If the user asks for something unsupported, the AI should not create a new component. It should list it under unsupported_requirements.

Example unsupported input:

Make me a board with temperature, humidity, GPS, and camera.

Expected extraction: JSON

{

"power": "USB-C 5V",

"connectivity": ["WiFi", "Bluetooth"],

"requested_sensors": ["temperature", "humidity"],

"selected_components": ["AHT20"],

"unsupported_requirements": ["GPS", "camera"]

}

The system should then say:

This prototype currently supports temperature, humidity, light, air quality, and pressure sensing. GPS and camera are outside the current showcase scope.

# 7. Recommended implementation stack

Use this simple stack:

Python

Streamlit

OpenAI API or local LLM

JSON templates

Graphviz or Mermaid diagrams

Static PCB/schematic images

Recommended folder structure:

pcb_ai_demo/

app.py

data/

supported_sensors.json

design_template.json

assets/

schematic_base.png

pcb_base.png

pcb_aht20.png

pcb_aht20_bh1750.png

pcb_all_sensors.png

prompts/

extraction_prompt.txt

# 8. Supported sensor JSON

Use this JSON file: JSON

{

"temperature": {

"component": "AHT20",

"function": "Temperature sensing",

"interface": "I2C"

},

"humidity": {

"component": "AHT20",

"function": "Humidity sensing",

"interface": "I2C"

},

"light": {

"component": "BH1750",

"function": "Ambient light sensing",

"interface": "I2C"

},

"air_quality": {

"component": "SGP30",

"function": "VOC / air quality sensing",

"interface": "I2C"

},

"pressure": {

"component": "BMP280",

"function": "Pressure sensing",

"interface": "I2C"

}

}

Important rule:

If both temperature and humidity are requested, use only one AHT20 component.

# 9. Prototype success criteria

The prototype is successful if it can show:

- A user enters a natural language board requirement.

- The AI extracts the requested sensing functions.

- The system maps the request to supported sensor components.

- The system generates a component list.

- The system generates a block diagram.

- The system generates a netlist-style connection summary.

- The system displays a schematic-style output.

- The system displays a PCB layout-style output.

- The design changes visually depending on selected sensors.

# 10. Non-goals

Do not implement:

- real PCB auto-routing

- arbitrary component selection

- datasheet parsing

- full KiCad file generation

- manufacturing-ready Gerber generation

- support for every sensor

- real electrical simulation

- real product certification logic

This is a showcase prototype only.

# 11. Final expected demo behavior

A stakeholder should see the following:

Input:

“Make me a USB-C powered indoor monitoring board with WiFi, Bluetooth, temperature, humidity, and light sensing.”

System output:

- Parsed requirement

- Selected sensors: AHT20 + BH1750

- Fixed architecture: USB-C + regulator + ESP32 + I2C bus

- Component list

- Netlist summary

- Schematic view

- PCB layout view with selected sensors highlighted

The final impression should be:

This prototype demonstrates that natural-language hardware requirements can be mapped into structured PCB design outputs using a controlled AI-assisted design pipeline.

In addition there will be a PCB design look close like this :